SECTION 06 20 00 – THERMALLY MODIFIED FINISH CARPENTRY

GENERAL

SUMMARY

Section Includes: Thermally modified wood [railings] [trim] [____].

ADMINISTRATIVE REQUIREMENTS

Pre-Installation Conference:

Attendance: [Architect,] [Owner,] [Contractor,] [Construction Manager,] [Design/Builder] installer, and related trades.

Review: Project conditions, manufacturer requirements, delivery and storage, staging and sequencing, and protection of completed work.

SUBMITTALS

Action Submittals:

Product Data: Manufacturer’s descriptive data and product attributes.

Samples: [Selection samples.] [Verification samples.]

Closeout Submittals:

Maintenance Data: Product data and related information required for Owner's maintenance of products furnished.

Extra materials: [__] linear feet of each profile.

QUALITY ASSURANCE

Installer Qualifications: Minimum [2] [__] years’ experience in work of this Section.

Mockups: Minimum [4] [ __] feet of each component. Approved mockups [may] [may not] remain as part of the Work.

DELIVERY, STORAGE AND HANDLING

Deliver, store, and handle products in accordance with manufacturer’s instructions.

PROJECT CONDITIONS

Do not install products when environmental conditions are outside of manufacturer’s recommended limits.

WARRANTY

Provide manufacturer’s 20 year warranty.

PRODUCTS

MANUFACTURERS

Contract Documents are based on products by Arbor Wood Co., www.arborwoodco.com.

Substitutions: [Refer to Division 01.] [Not permitted.]

MATERIALS

Thermally Modified Wood:

Species: Ash.

Size: [Nominal 1 x 2 inches, actual 0.75 x 1.5 inches.]

- [Nominal 1 x 4 inches, actual 0.75 x 3.5 inches.]

- [Nominal 1 x 6 inches, actual 0.75 x 5.5 inches.]

- [Nominal 1 x 8 inches, actual 0.75 x 7.25 inches.]

- [Nominal 2 x 2 inches, actual 1.5 x 1.5 inches.]

- [Nominal 2 x 4 inches, actual 1.5 x 3.5 inches.]

- [Nominal 2 x 6 inches, actual 1.5 x 5.5 inches.]

- [Nominal 2 x 8 inches, actual 1.5 x 7.25 inches]

- [Nominal 2 x 10 inches, actual 1.5 x 9.25 inches.]

- [Nominal 2 x 12 inches, actual 1.5 x 11.25 inches.]

Collection: Natrl.

Profile: Eased edge.

Finish: [No finish.] [Pre-oiled with Cutex Extreme.]

**** OR ****

Thermally Modified Wood:

Species: Pine.

Size: [Nominal 1 x 2 inches, actual 0.75 x 1.5 inches.]

- [Nominal 1 x 4 inches, actual 0.75 x 3.25 inches.]

- [Nominal 1 x 6 inches, actual 0.75 x 5.25 inches.]

- [Nominal 1 x 8 inches, actual 0.75 x 7.25 inches.]

- [Nominal 2 x 2 inches, actual 1.5 x 1.5 inches.]

- [Nominal 2 x 4 inches, actual 1.5 x 3.25 inches.]

- [Nominal 2 x 6 inches, actual 1.5 x 5.25 inches.]

- [Nominal 2 x 8 inches, actual 1.5 x 7.25 inches]

- [Nominal 2 x 10 inches, actual 1.5 x 9 inches.]

- [Nominal 2 x 12 inches, actual 1.5 x 11 inches.]

Collection: Natrl.

Profile: Eased edge.

Finish: Pre-oiled with [clear Cutex oil.] [Cutex Extreme.]

**** OR ****

Thermally Modified Wood:

Species: Poplar.

Size: [Nominal 1 x 2 inches, actual 0.75 x 1.5 inches.]

- [Nominal 1 x 4 inches, actual 0.75 x 3.5 inches.]

- [Nominal 1 x 6 inches, actual 0.75 x 5.5 inches.]

- [Nominal 1 x 8 inches, actual 0.75 x 7.25 inches.]

Collection: Natrl.

Profile: Eased edge.

Finish: Pre-oiled with [clear Cutex oil.] [Cutex Extreme.]

**** OR ****

Thermally Modified Wood:

Species: Douglas Fir.

Size Nominal 4 x 4 inches, actual 3.375 x 3.375 inches.

Profile: Eased edge.

Finish: No finish.

ACCESSORIES

Fasteners: Type 305 stainless steel screws.

End Sealer: Cutex or Achor Seal.

Finish: Cutex Extreme [with Cutex Colortone added, [____] color [to be selected from manufacturer’s full color range].

EXECUTION

INSTALLATION

Install in accordance with manufacturer’s instructions.

Apply end sealer to ends prior to installation.

Apply finish to exposed surfaces to improve weatherability.

END OF SECTION