SECTION 07 46 23 – THERMALLY MODIFIED EXTERIOR WOOD CLADDING

GENERAL

SUMMARY

Section Includes: Thermally modified exterior wood cladding.

ADMINISTRATIVE REQUIREMENTS

Pre-Installation Conference:

Attendance: [Architect,] [Owner,] [Contractor,] [Construction Manager,] [Design/Builder] installer, and related trades.

Review: Project conditions, manufacturer requirements, delivery and storage, staging and sequencing, and protection of completed work.

SUBMITTALS

Action Submittals:

Product Data: Manufacturer’s descriptive data and product attributes.

Samples: [Selection samples.] [Verification samples.]

Informational Submittals:

Certificate of Compliance: Third party certification that products meet specified fire-resistance requirements.

Closeout Submittals:

Maintenance Data: Product data and related information required for Owner's maintenance of products furnished.

Extra materials: [__] linear feet of each profile.

QUALITY ASSURANCE

Installer Qualifications: Minimum [2] [__] years’ experience in work of this Section.

Mockup: Minimum [4 x 4] [__ x  __] feet. Approved mockup [may] [may not] remain as part of the Work.

DELIVERY, STORAGE AND HANDLING

Deliver, store, and handle products in accordance with manufacturer’s instructions.

PROJECT CONDITIONS

Do not install products when environmental conditions are outside of manufacturer’s recommended limits.

WARRANTY

Provide manufacturer’s 20 year warranty.

PRODUCTS

MANUFACTURERS

Contract Documents are based on products by Arbor Wood Co., www.arborwoodco.com.

Substitutions: [Refer to Division 01.] [Not permitted.]

REGULATORY REQUIREMENTS

Fire Retardant Treated Wood:

Class A rated, tested to ASTM E2768

30 minute burn rated, tested to NFPA 703 ignition resistant.

MATERIALS

Thermally Modified Exterior Wood Cladding:

Species: Ash.

Profile: Tongue and groove, end matched.

Size: [0.75 x 3.25 inches; 3 inch face dimension.] [0.75 x 5.25 inches; 5 inch face dimension.] [0.75 x 7.25 inches, 7 inch face dimension.]

Installation method: [Blind fasteners.] [Screws.]

Collection: Natrl.

Finish: No finish.

Surface Texture: Smooth

**** OR ****

Thermally Modified Exterior Wood Cladding:

Species: Ash.

Profile: Tongue and groove, end matched.

Size: [0.75 x 3.25 inches; 3 inch face dimension.] [0.75 x 5.25 inches; 5 inch face dimension.] [0.75 x 7.25 inches, 7 inch face dimension.]

Installation method: [Blind fasteners.] [Screws.]

Collection: Elmnt.

Surface texture: Light Brushed

Finish: [Sand] [Stone] [Iron] [Quartz] [Onyx]

- **** OR ****

Thermally Modified Exterior Wood Cladding:

Species: Ash.

Profile: Fluted tongue and groove.

Size: 1 x 3.25 inches; 3 inch face dimension.

Installation method: [Blind fasteners.] [Screws.]

Collection: Natrl.

Finish: [No finish.] [Pre-oiled with Cutex Extreme.]

Surface Texture: Smooth.

**** OR ****

Thermally Modified Exterior Wood Cladding:

Species: Southern Yellow Pine [Mixed Grain] [Vertical Grain].

Profile: Tongue and groove with nickel gap.

Size: [0.75 x 3.25 inches; 3 inch face dimension.] [0.75 x 5.25 inches; 5 inch face dimension.] [0.75 x 7.25 inches; 7 inch face dimension.]

Installation method: [Blind fasteners.] [Screws.]

Collection: Natrl.

Finish: Clear Cutex oil.

Surface texture: Smooth.

**** OR ****

Thermally Modified Exterior Wood Cladding:

Species: Southern Yellow Pine [Mixed Grain] [Vertical Grain].

Profile: Tongue and groove.

Size: [0.75 x 3.25 inches; 3 inch face dimension.] [0.75 x 5.25 inches; 5 inch face dimension.] [0.75 x 7.25 inches, 7 inch face dimension.]

Installation method: [Blind fasteners.] [Screws.]

Collection: Elmnt.

Surface texture: Light Brushed

Finish: [Sand] [Stone] [Iron] [Quartz] [Onyx]

- **** OR ****

Thermally Modified Exterior Wood Cladding:

Species: Southern Yellow Pine [Mixed Grain] [Vertical Grain].

Profile: Fluted tongue and groove.

Size: 1 x 5.25 inches; 5 inch face dimension.

Installation method: [Blind fasteners.] [Screws.]

Collection: Natrl.

Finish: [No finish.] [Pre-oiled with Cutex Extreme.]

Surface texture: Smooth.

**** OR ****

Thermally Modified Exterior Wood Cladding:

Species: Southern Yellow Pine.

Profile: Tongue and groove with nickel gap.

Size: 0.75 x 5.25 inches; 5 inch face dimension.

Installation method: [Blind fasteners.] [Screws.]

Collection: Brnsh.

Surface texture: Charred.

Finish: [Burned, black oil.] [Burned and brushed, black oil.] [Burned and brushed, brown oil.]

**** OR ****

Thermally Modified Exterior Wood Cladding:

Species: Poplar.

Profile: Tongue and groove with nickel gap

Size: [0.75 x 3.25 inches; 3 inch face dimension.] [0.75 x 5.25 inches; 5 inch face dimension.] [0.75 x 7.25 inches; 7 inch face dimension.]

Installation method: [Blind fasteners.] [Screws.]

Collection: Natrl.

Finish: Clear Cutek oil.

Surface texture: Smooth.

**** OR ****

Thermally Modified Exterior Wood Cladding:

Species: Poplar.

Profile: Tongue and groove.

Size: [0.75 x 3.25 inches; 3 inch face dimension.] [0.75 x 5.25 inches; 5 inch face dimension.] [0.75 x 7.25 inches, 7 inch face dimension.]

Installation method: [Blind fasteners.] [Screws.]

Collection: Elmnt.

Surface texture: Light Brushed

Finish: [Sand] [Stone] [Iron] [Quartz] [Onyx]

ACCESSORIES

Attachment System: Grad Hidden Click-On System.

Furring Strips: [5/16] [1/2] [3/4] inch thick x 1-1/2 inches wide.

Fasteners: Stainless steel, type recommended by cladding manufacturer.

End Sealer: [Cutek.] [Anchor Seal.]

Finish: Cutex Extreme [with Cutex Colortone added, [____] color [to be selected from manufacturer’s full color range.

EXECUTION

INSTALLATION

Install in accordance with manufacturer’s instructions.

Apply end sealer to ends prior to installation.

Apply finish to exposed surfaces.

END OF SECTION